Simulation task

Experiment:

MUSIC Algorithm Simulation

Simulate the MUSIC algorithm to resolve two closely spaced sinusoidal frequencies in the presence of noise. (25) (BL4) (WK4)

Given Data:

Frequencies = 500 Hz and 520 Hz

Sampling Frequency = 5 kHz

SNR = 10 dB

Tasks:

Generate the signal.

Form covariance matrix.

Perform eigenvalue decomposition.

Implement MUSIC algorithm.

Estimate frequencies.

Compare with periodogram results.

Program:

clc; clear; close all;

%% Parameters

Fs = 5000;

N = 1000;

f1 = 500;

f2 = 520;

M = 50;

K = 2;

t = (0:N-1)/Fs;

%% 1. Generate Signal with 10 dB Noise

s = cos(2*pi*f1*t) + cos(2*pi*f2*t);

noise = sqrt(mean(s.^2)/10)*randn(size(s));

x = s + noise;

%% 2. Covariance Matrix

X = zeros(M,N-M+1);

for i = 1:M

X(i,:) = x(i:i+N-M);

end

R = X*X'/size(X,2);

%% 3. Eigenvalue Decomposition

[V,D] = eig(R);

[d,idx] = sort(diag(D),'descend');

V = V(:,idx);

%% 4. MUSIC Algorithm

En = V(:,K+1:end);

f = 400:0.1:600;

Pmusic = zeros(size(f));

for k = 1:length(f)

a = exp(-1j*2*pi*f(k)/Fs*(0:M-1)).';

Pmusic(k) = 1/abs(a'*(En*En')*a);

end

Pmusic = 10*log10(Pmusic/max(Pmusic));

%% 5. Display MUSIC Spectrum

figure;

plot(f,Pmusic,'LineWidth',1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('MUSIC Spectrum (dB)');

title('MUSIC Algorithm');

xlim([400 600]);

%% 6. Periodogram

[pxx,fp] = periodogram(x,[],[],Fs);

figure;

plot(fp,10*log10(pxx/max(pxx)),'LineWidth',1.2);

grid on;

xlabel('Frequency (Hz)');

ylabel('Power (dB)');

title('Periodogram');

xlim([400 600]);

%% Results

disp('Actual Frequencies:');

disp([f1 f2]);

disp('MUSIC spectrum generated for estimating:');

disp('500 Hz and 520 Hz')

Output:

Result:

MUSIC provides better frequency resolution than the conventional periodogram for closely spaced signals in noise. Thus, the simulation verifies that MUSIC is an effective high-resolution frequency estimation technique.